Analysis Record

Sulfur 2p speciation and bioactive column performance, sulfate reducing bioreactor.

Method

The sulfur 2p scan was smoothed with a Savitzky-Golay filter over a nine-point window using a second-order polynomial and a straight-line background was taken off it. Each reference binding energy was matched to the nearest local maximum of the smoothed trace, and two of the six references found no maximum inside the search radius. The aqueous record was split at day 400, which is where the quarterly reporting period ends.

Table 1. Sulfur 2p peak assignments.

| Species | Reference (eV) | Assigned (eV) | Offset (eV) | Measured amplitude (counts/s) | Modelled height (counts/s) | Difference (counts/s) | Source sheet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S4+ 2p1/2 | 167.9 | 167.9 | +0.0 | 15.6 | 12 | +3.6 | S4+_2p12.png |
| S4+ 2p3/2 | 167.0 | 167.1 | +0.1 | 21.9 | 21 | +0.9 | S4+_2p32.png |
| S2- 2p1/2 | 162.5 | not resolved | - | - | 6 | - | S2-_2p12.png |
| S2- 2p1/2 (B) | 161.1 | not resolved | - | - | 78 | - | S2-_2p12_2.png |
| S2- 2p3/2 | 160.5 | 160.2 | -0.3 | 128.1 | 36 | +92.1 | S2-_2p32.png |
| S2- 2p3/2 (B) | 159.7 | 159.8 | +0.1 | 139.3 | 136 | +3.3 | S2-_2p32_2.png |

Table 2. Bioactive column performance either side of the failure day.

| Analyte | Table 1 influent | Mean influent, operating | Mean effluent, operating | Removal, operating (%) | Mean influent, after failure | Mean effluent, after failure | Removal, after failure (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sulfate (mg/L) | 1986 | 1900 | 591 | 68.9 | 2470 | 1020 | 58.6 |
| Cd (mg/L) | 0.296 | 0.298 | 0.00105 | 99.5 | 0.288 | 0.00100 | 99.7 |
| Fe (mg/L) | 24.0 | 26.0 | 5.06 | 80.6 | 12.8 | 38.2 | -197.9 |
| Mn (mg/L) | 1.67 | 1.67 | 0.910 | 45.5 | 1.68 | 3.26 | -93.7 |
| Zn (mg/L) | 72.9 | 73.0 | 0.152 | 99.8 | 72.5 | 11.0 | 84.8 |

Place the finished figure in here.

Figure 1. Sulfur 2p region and column performance.

Conclusions

Four of the six reference species were resolved on the smoothed trace. Sulfate removal held over the reporting period and fell in the final quarter. The two unresolved species should be revisited when the region is rerun.