| PROJECT ACB Windsor - Clinical Suites | PROJECT NO. 21-515 |
| --- | --- |
| DESCRIPTION Supply and installation of Clean Utility Distribution Systems (Fermentation Room) | DATE October 29, 2021 |

# Invitation

Clinical Suites

Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room)

## Contractors are requested to submit detailed information on proposed scope of work. Proposals can be sent by email or delivered in person.

# Proposal Form

## Proposals will be submitted on the Proposal Form provided.  All parts of the Proposal Form must be properly completed.

## Contractor shall include in proposal submittal a clear List identifying all requirements, materials, and/or performance identified within this document that the Contractor’s proposed equipment does not meet as a minimum. Contractor is cautioned that if any item or equipment performance is later found to fail to meet the requirements contained within this document without being clearly notified in the original proposal submittal on the deficiencies list it will be considered a deficiency and shall be rectified by the Contractor, at the Contractors expense, to the Owner’s satisfaction.

# Form of Agreement

## The Form of Agreement will be owner issued purchase order.

# Owner’s COVID-19 Policy

## As of October 29, 2021, all visitors and Contractors to ACB sites must be fully vaccinated, provide adequate proof of vaccination, and complete the “ACB Visitors and Contractors Screening Form” (provided at the end of this this document).

## In addition, they must follow the public health rules in place for travel to PEI or NS, all public health guidelines in place for movement within the applicable province, and all COVID-19 protocols in place at ACB’s facilities.

## ACB COVID-19 protocols in place may include the following:

### Wearing a mask and maintaining physical distance (two metres) while on-site.

### Limiting contracts to select individuals while on-site.

### Self-isolating when not on-site, including insolation, meal, travel, and making support plans in case visitor develops COVID-19 symptoms.

### Following COVID-19 Public Health testing protocols.

### Working in isolated environments for the duration or a portion of their visit.

## The “ACB Visitors and Contractors Screening Form” must be provided prior to arranging the visitor or Contractor site access, and updated in cases of longer-term visits and/or work, such as but not limited to:

### If the information on the form provided prior to the visit is older than three (3) days.

### Every Monday.

### After being off-site for more than one (1) day.

### When circumstances have changed affecting the visitor/Contractor’s responses to the form, in which case ACB would require re-evaluation of site access.

## If access is granted, the visitor/Contractor must sign the screening form provided by the on-site contact and then return is to reception to receive their visitor’s pass. No new visitor’s passes will be issued unless accompanied by a completed screening form. Reception will file a copy.

# Site Visit

## Prior to submitting a Bid, Contractors shall visit the project site. Tender submissions shall reflect all site conditions.

## A site visit is mandatory to bid this project, in-person and virtual options are available.

### In-person site visit is scheduled for 2:00pm, Friday November 5, 2021. Meet the Engineer at the Front Entrance at that time..

### If you are unable to attend in-person, a virtual site visit will be run in parallel with the in-person site visit (Microsoft Teams stream), or an alternate time for a virtual site visit can be arranged with ACB and MCP personnel.

### A 3D scan of the Fermentation suite has also been made available to assist in reviewing site conditions (link provided in the RFP distribution email).

# Schedule

## Contractor shall submit at minimum a Level 1 schedule with the proposal indicating start and end dates for the following activities, assuming contract award date of Tuesday, November 16, 2021.

### Shop drawing generation and approval

### Material procurement

### (Pre) Fabrication

### Site installation

### Turn Over Package delivery

### Commissioning/deficiencies resolution

### Training

## Site installation schedule is specified in the schedule template located at the end of this document. This timeframe is based on completing the work during a process area shutdown and in coordination with other work that will be completed both before and after the scope of work defined in this RFP.

## If 10- or 12-hours shifts are required in order to maintain the requested installation schedule; please identify this during the bidding stage and include appropriate pricing to accommodate overtime hours (if applicable).

## Weekend work on site (if required) is acceptable to meet indicated site installation schedule.

## For the duration of the project (outside site installation) the contractor must provide schedule updates on a bi-weekly basis and identify schedule risks/issues proactively and immediately as they are identified between the bi-weekly schedule updates.

## For the duration of site installation work the contractor must provide both daily updates on work conducted/progress in the form of a daily report, and a weekly progress update report no later than EOD Tuesday following the preceding week (Sunday to Saturday).

## In the event long lead items (valves, etc.) are not available in time for the installation period, placeholder spools must be included in the piping system and the remaining long-delivery items can be shipped loose to site and installed (tri-clamp, flange, etc.) by ACB staff.

# Permits, Taxes and Regulations

## All permits and fees required for the proper completion and inspection of the work herein specified will be paid for by the Contractor.

## In the absence of any provisions contained herein, the applicable Provincial Codes, or the latest edition of the National Building Code of Canada will govern.

# Scope of Work

## For comprehensive scope of work definition refer to design documents, specifications, and scope of work definition table.

## Where any contradicting conditions or requirements have been included in both this document and the provided design specifications, the more rigorous or stringent condition shall be adhered to.

## Contractor shall supply and install equipment, components and material as specified. Site installation of Owner supplied equipment shall be performed by this Contractor in accordance with Vendor’s installation instructions. Contractor is responsible for final clean utility tie-ins to the following Owner supplied equipment:

### 100L CSR Fermenter & TCU package

### 1000L CSR Fermenter & TCU package

## The following design documents shall be considered as part of this RFP to provide guidelines for scope of work required:

### Drawing: 2021.10.29 – 20.2-M101 - (PID-W34) Clean Air Distribution 1 - IFP

### Drawing: 2021.10.29 – 20.3-M101 - (PID-W69) Oxygen Storage and Distribution System - IFP

### Drawing: 2021.10.29 - 20.5-M101 - (PID-W68) Harvest & DSP Suites CIP/Buffer/Transfer - IFP

### Drawing: 2021.10.29 - 20.2-M201 - Clean Air Plans - IFP

### Drawing: 2021.10.29 - 20.3-M201 - Oxygen System Plans - IFP

### Drawing: 2021.10.29 - 20.4-M201 - Process Vent Plans - IFP

### Drawing: 2021.10.29 - 20.5-M201 - Process Piping Plans - IFP

### Component Tag List: 2021.10.29 – Contract 20.0 – CTL – IFP

### Supplementary Instructions and Addendums (as required)

## The following is a general outline of the scope of work and considerations for pricing:

### Installation of Clean Air Distribution System.

### Installation of Oxygen Distribution System.

### Installation of Process Piping Transfer Systems.

### Installation of Process Vent Piping System.

#### Include piping and all necessary fittings to connect components and equipment.

#### Any reduction in pipe size required for connection to equipment or components shall be made at the equipment (all reduced pipe runs shall be avoided).

## Pipe and duct routing as shown and described within this RFP and referenced design documents to be coordinated on site with MCP Consultants and Owner’s representatives, and any additional costs related to site conditions or field fitting that would have been discovered during a thorough site investigation during bidding shall be the responsibility of the Contractor. Contractor to include for all scaffolding necessary for equipment and piping installation (as required).

## Pipe and conduit support and support structure can be supported from the ground, roof, the mezzanine, and adjacent structure. Structural design, fabrication, and installation is by this contractor and their subcontractors as part of this contract.

## Contractor shall supply start-up assistance and 1-year warranty for all work provided (details in section ‎14. Guarantee).

## Contractor shall include for a training allowance to assist with equipment commissioning with Owner staff.

## To minimize in-suite and on-site work to expedite installation, the Contractor shall make all possible efforts to measure site conditions and pre-fabricate as much piping as possible in advanced of the installation period. Final trim allowance and field-fit should be allowed for as final/exact placement of equipment may vary from 3D models.

## Welding will be completed either within the facility in a designated area, or inside a “clean welding” trailer provided by the General Contractor or added subsequently to the scope of this Contract as a change order. No allowance for a welding trailer is required at this time.

## If possible, material and office trailer should be combined to minimize area required for trailer placement on site (site is constrained).

# Alternate Products

## Alternative price indicates prices that are offered or requested in Specification Section on components or systems described in comparison with base bid and listed as “Alternate Products”.  Unless requested in Specification Section, alternative price submission is not mandatory.  When submitting pricing for Alternate Products the Contractor must include the list identified in section ‎2.2 above.

# Shop Drawings

## The successful Contractor shall submit electronic shop drawings for all components for review. Shop drawings shall be first quality reproductions with all details, specifications, and installation instructions identified. Fax copies are not acceptable. Work shall not commence until Engineer’s approval has been obtained.

## Provide capacity and performance data indicating specified conditions on all equipment.

## Provide manufacturers data sheets including materials of construction.

## Provide cut sheets of all proposed equipment and components.

# Operations and Maintenance Manuals

## Successful Contractor shall provide three (3) copies of complete operation and maintenance manuals as well as one (1) electronic copy in PDF format.

## Prepare a maintenance schedule which will advise the Owner’s staff what maintenance must be done and the suggested intervals at which it should be done.

## Submit one (1) copy at least two weeks before instruction to Owner is commenced.

## Ensure that the terminology used in various sections of the manual is consistent.

## Each manual shall contain the following information:

### Table of Contents

### Description of each major component of system

### Complete sets of page size equipment Shop Drawings

### Equipment manufacturer's installation, start-up, and operation manuals

### Equipment manufacturer's recommended spare parts lists

### Equipment wiring diagrams

### Lubrication schedule for all equipment

### Equipment identification list with serial numbers

### Copies of all test records, and start-up reports

### Final balancing reports

### Control drawings, sequences of operation (as applicable)

### Extended warranty documentation (as applicable)

## Manuals are to be completed and approved in time to be used during instruction of operating staff.

# Training

## Instruct operating staff on maintenance, adjustment, and operation of mechanical equipment.  Instruct staff on changes or modification in equipment made under terms of guarantee.

## Provide instruction during regular work hours prior to acceptance and turn-over to operating staff for regular operation.

# Duration and Acceptance of Offer

## Tenders shall remain open for acceptance and shall be irrevocable for a period of thirty (30) days after the Proposal date.

## The Owner reserves the right to accept or reject any or all offers.

# Guarantee

## This Contractor shall guarantee their work, equipment, components, and materials free from defects for a period of one (1) year, unless specifically noted otherwise, after final acceptance of such equipment by the Owner and shall make good all defects other than normal wear and tear during the life of the guarantee. This Contractor shall guarantee all work and equipment supplied by them to work quietly and satisfactorily and to accomplish the work for which it was installed during the life of the above guarantee. At any time during this period, they shall make any necessary changes and adjustments, or replacements, to accomplish this at their own expense.

# Protection and Security

## The Contractor shall be responsible for the protection of all work, material and equipment of other contractors and Owner's property as necessary during the performance of this contract. Any damage resulting from failure to comply with this requirement is the complete responsibility of the Contractor.

## The Contractor shall be responsible to secure and safely stored all tools and equipment, whether those are supplier/subcontractor-owned, project materials or equipment stored on site.  All materials and equipment shall be secured during and at the end of each work shift.  Neither MCP Consultants or the Owner shall be held responsible to cover the costs of any of these losses including any decreased or lost labor productivity.

# Other Contracts

## Separate Contracts are being awarded for other sections of the Project. Contractors shall be fully knowledgeable and anticipate the work of other contractors (through regular coordination meetings/discussions) wherein work of this Contract is affected. The Contractor shall co-operate, co-ordinate and interface work of this Contract with work of all other Contracts for the Project.

# List of Subcontractors

## Contractor to provide a list of names of all subcontractors the Contractor proposes to use to perform work when requested by MCP Consultants or the Owner.

# Discrepancies, Omission, Etc.

## Notify MCP Consultants of discrepancies in, or omissions from the drawings, specifications, or other documents. MCP Consultants will give written instructions, explanations, or clarification in the form of Change Orders or Site Instructions.

# Addenda

## Addenda may be issued during the tender period. All addenda become part of the Contract Documents. Include costs as applicable in the tender price.

## Bidders are to complete Appendix A as acknowledgement of receipt of addenda.

# Health & Safety

## Contractors shall submit the following documentation along with their bid submission

### Good standing with the NS Workers' Compensation Board (Clearance letter).

### Safety Certification, CSNS (formerly NSCSA) Letter of Good Standing or equivalent.

### Current General Liability insurance coverage.

## ln addition, The Contractor, prior to starting work, shall complete a pre job hazard assessment which identifies all hazards and potential hazards and the actions taken to control them. Contractors may be required to submit a formal "Safety Plan" for any major project.

## The Contractor shall provide a representative to participate in all work site safety meetings and play active roles as members of the work site JOHS committee (if such committee is mandated by the OH&S Act). This does not relieve The Contractor of any duty to maintain the responsibilities they have under the Occupational Health and Safety Act and any other acts which may be applicable under law.

## The Contractor shall conform to all Health and Safety requirements including, but not limited to, weekly inspections, Weekly Toolbox talks, PPE, General rules & Discipline.

## The Client (ACB) or representative (MCP Consultants) will be provided with documentation to verify the ongoing activity related to health and safety as dictated by corporate policy and the nature and complexity of the project.

# Future Work

## A future package for clean utilities related to this same project (nearby suites in the same facility) will be issued in ~3 weeks time. This work will occur starting in early 2022 and will be similar, but slightly larger in scope, to the scope of work identified in this bid package. The goal is to have this future work overlap with the completion of this work package.

# Experience/Training Records

## Resume, experience records, and training records of key personnel who will be directly involved with this project (or several individuals if final selection of trades for the project is not completed), if awarded, must be provided with the bid submission. ASME-BPE stainless tubing installation and documentation is a specialized skillset and documented training and/or past proven experience is a requirement for this project. Key personnel for which information is required:

### QA/QC technicians/managers/staff

### Welders

### Fitters

### Project Managers

### Inspectors (3rd party is acceptable)

## On award of contract, welders from out of province will need to complete an Inter-Provincial welder qualification card transfer, and all required procedures.

# Union Requirements

## If an out-of-province Contractors’ trades are unionized, Contractor shall be entirely responsible for identifying all requirements for bringing out-of-province trades to site including, but not limited to, requirements for local labour support of out-of-province workers. If the contractor fails to identify and understand local union requirements that are later identified and create additional costs, these costs will be borne entirely by the Contractor, at no expense to the Client/project.

# 3D Scans

## A complete scan of the Fermentation suite is available on request. MCP Consultants and ACB do not guarantee the accuracy of the scan; Neither organization accepts liability for issues/errors resulting from routing/design completed based on the 3D scan. All measurements/routing should be confirmed by the Contractor on site or adequate trim allowance maintained to accommodate any variances from the scan/drawings to actual site conditions. It is solely the Contractor’s responsibility to fit up and install a completed system accommodating for site conditions.

Clinical Suites

Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room)

PROPOSAL PACKAGE

THE FOLLOWING CONSTITUTES A COMPLETE PROPOSAL PACKAGE.  ALL THESE ITEMS MUST BE INCLUDED IN THE PROPOSAL SUBMISSION TO ENABLE THE OWNER TO DECLARE THE SUBMISSION AS BEING COMPLETE AND VALID.

Proposal form as provided		1 copy complete and signed.

Level 1 Schedule			1 copy to be submitted by bidders.

Appendix A				1 copy to be submitted by bidders (as required).

Additional Information			Submit on Contractor forms as required.

Clinical Suites

Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room)

PROPOSAL FORM

CONTRACT TENDER:

SUBMITTED BY:

Contractor

Address

Telephone					Fax

DATE:

FOR:		Clinical Suites

Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room).

Having examined the specifications and all documents contained within the Request for Proposal, as well as any addenda issued, WE HEREBY OFFER to provide work, material and components, including equipment start-up as necessary for the full and proper completion of the  Clinical Suites  - Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room), including all prime cost allowances, but not including HST or any additional or deductible allowances or taxes which may be applicable subsequent to this date, which shall be payable by or to the Owner, in accordance with the above mentioned documents, for the sum of:

Dollars

($                                                                        ) in lawful money of Canada.

Note: HST is not to be included in the above tendered price.

Completion Date:                                             	 weeks from receipt of approval to proceed.

Hours and shift acceptance (REQUIRED)

On site installation hours are planned as follows (to meet indicated installation schedule):

Days/week

Hours/day

Indicate if the following shift arrangements (to accommodate 8/10/12 hour shifts as is deemed required to meet schedule) are acceptable by writing “yes” or “no” beside each shift arrangement:

7am start:

3pm start:

5pm start:

7pm start:

Price breakdown (REQUIRED):

Provide pricing breakdown per below with total of all lines below equaling total submitted price (i.e., all pricing must be assigned to the below categories):

| Category | Pipe Spec | Pipe Size Range | Total Length | Labour Cost | Material Cost |
| --- | --- | --- | --- | --- | --- |
| 20.2 - Clean Air Distribution | Ps | ½” to 1-1/2” | 220 |  |  |
| 20.3 - Clean Oxygen Distribution | Ps | ¾” to 2” | 145 |  |  |
| 20.4 - Process Vent System | Pa | 2” to 3” | 74 |  |  |
| 20.5 - Process Piping System | Ps | 1” | 60 |  |  |
| Equipment Rentals (Cranes, Lifts, etc.) |  |  |  |  |  |
| Startup Support |  |  |  |  |  |
| Testing & Documentation |  |  |  |  |  |
| Project Management |  |  |  |  |  |
| General Conditions (Shipping/Crating, Mobilization, Cleanup, etc.) |  |  |  |  |  |
| Other (Specify): |  |  |  |  |  |
| Other (Specify): |  |  |  |  |  |
| Other (Specify): |  |  |  |  |  |

Labour Prices (REQUIRED)

In the event time and material or fixed price changes are required the undersigned Bidder does hereby Bid and offer to enter into a Contract to perform and complete any unit price work and provide all necessary labour, plans, tools, materials, and equipment, including its overhead and profit as set forth and in strict accordance with the Specifications, Drawings, and other Contract Documents and to do all therein set forth including all taxes for the following unit pricing excluding HST: Labour Pricing must be submitted for the following areas with your stipulated sum price Proposal submittal.

| Category | Apprentice | Journeyman | Foreman |
| --- | --- | --- | --- |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |
|  | $ /hr | $ /hr | $ /hr |

Specify markup percentage to be applied to change order costs. This percentage must be inclusive of all markups (insurance, bonding, profit, etc.) as specified in the specifications included with this RFP.

Changes below $3000, markup percentage: 	 		%

Changes $3000 and greater, markup percentage:  		%

If night shift premiums are required, specify the premium and at what time night-shift hours apply:

Night-shift Premium:

Night-shift hours starting hour:

Trailer Requirements (REQUIRED)

Specify the number and size of any trailers required to complete work on site. If possible, material and office trailer should be combined to minimize area required for trailer placement on site (site is constrained):

| Description | Size |
| --- | --- |

Acceptance

In submitting this Proposal, we recognize the necessity to complete the information requested by any appendices, as well as the right of the Owner to reject all Proposals or to accept any Proposal at the price submitted.

If we are notified of the acceptance of this Proposal within seven days, we will:

  - Enter into a formal Contract Agreement with the Owner.

  - .2	Agree to execute the contract according to schedule as provided under Section ‎6. identified on page 2 of this Proposal Form.

COMPANY (PRINT):

PER (PRINT):

SIGNATURE:

| Task | Start Date | End Date | Additional Comments |
| --- | --- | --- | --- |
| Shop drawing generation and approval |  |  |  |
| Material procurement |  |  |  |
| (Pre) Fabrication |  |  |  |
| Site installation | Jan 12, 2022 | Jan 31, 2022 |  |
| Turnover package delivery |  |  |  |
| Commissioning/Deficiencies Resolution |  |  |  |
| Training |  |  |  |

Schedule

Assume contract award date of Tuesday, November 16, 2021.

ACB Windsor - Clinical Suites

Contract 20: Supply and installation of Clean Utility Distribution Systems (Fermentation Room)

APPENDIX A - ADDENDA

ACKNOWLEDGMENT OF RECEIPT OF ADDENDA

Addendum No. & Issue Date:                                                         		Initial:

Addendum No. & Issue Date:                                                         		Initial:

Addendum No. & Issue Date:                                                         		Initial:

Addendum No. & Issue Date:                                                         		Initial:

Addendum No. & Issue Date:                                                         		Initial:

Date of request:  			 	ACB Onsite Contact:

Name:  						Company:

Phone number:					Email:

Travelling from (If applicable):  		              	ACB Site to access:

Please complete either of the following sections which applies to your situation:

| IN-PROVINCE (LOCAL) VISITORS / CONTRACTORS Please respond to the following questions: |  |
| --- | --- |
| - I am fully vaccinated as evidenced by provision of copy of: PEI Vax Pass or NS COVID-19 Proof of Vaccination. | - Yes ☐No |
| - I have no COVID-19 symptoms, have not been in contact with someone with active COVID-19 or with someone in isolation within 14 days prior to arriving on-site. | - Yes ☐No |
| - I understand physical distancing, will wear a mask, and follow ACB’s COVID-19 protocols while on site. | - Yes ☐No |

| OUT-OF-PROVINCE VISITORS / CONTRACTORS Please respond to the following questions: |  |
| --- | --- |
| - I am fully vaccinated as evidenced by provision of copy of: PEI Pass or NS Safe Check-In Form. | - Yes ☐No |
| - I have no COVID-19 symptoms, have not been in contact with someone with active COVID-19 or with someone in isolation within 14 days prior to arriving on-site. | - Yes ☐No |
| - I understand physical distancing, will wear a mask, and follow ACB’s COVID-19 protocols while on site. | - Yes ☐No |

Visitor/Contractor Signature	ACB Onsite Contact Signature

Note: Completing this form does not authorize visitation. Onsite contact will use the information on this form to request executive management authorization.  All international travellers who are permitted to enter Canada must follow the federal requirements and complete approval process for the PEI Pass and/or the Nova Scotia Safe Check-In form prior to entering PEI and/or NS.